卫星仿真平台工程级模型验证测试细则

/ satellite-simulation-platform 0.5.3 / 0.5.3

# 1. 文档范围与结论

本文档面向  工程级模型验证测试体系。测试范围覆盖环境、静态质量、标定证据、能力注册、部件级模型、分系统、整星代表场景、Agent 规划、Golden Set 和发布收口。

本细则以机器可执行矩阵 configs/verification/engineering_verification_matrix.json 为权威来源；每个测试项均给出可直接执行的命令、判读参数和通过/失败规则。

边界： 验证当前已注册且已实现的公开 Runner、事件/退化机制和 Agent 链路，不声称覆盖未建模物理、真实标定或飞行认证。

| 统计项 | 数值 |
| --- | --- |
| 矩阵版本 | .engineering-verification-matrix.v1 |
| 测试项总数 | 437 |
| 部件数量 | 24 |
| 分系统数量 | 6 |
| 部件级测试项 | 392 |
| 分系统测试项 | 12 |
| 环境/静态/模型/标定/Agent/发布测试项 | 33 |

# 2. 通用执行方式

推荐从工程根目录执行以下命令。Linux/WSL：

export PYTHONPATH=.:src
python scripts/run_engineering_verification_matrix.py
 --statspython scripts/check_engineering_verification_matri
x.pypython scripts/run_engineering_verification_matrix.py --level component --execute-all --output-dir reports/既有工程版本_c
omponent_matrixpython scripts/run_engineering_verification_matrix.py --level subsystem --execute-all --output-dir reports/既有工程版本_subsystem_matrix

单项测试命令格式：

PYTHONPATH=.:src python scripts/run_engineering_verification_matrix.py --test-id <TEST_ID> --output reports//<TEST_ID>.json

# 3. 判读通用规则

| 结果 | 含义 | 研判规则 |
| --- | --- | --- |
| PASS | 测试项满足判读参数和通过规则 | 可进入下一层测试；仍需保留 JSON 证据。 |
| FAIL | 命令或自动判读失败 | 不得以人工解释替代；需定位 reason、stderr、Run Bundle 或 QoI。 |
| INCONCLUSIVE | 执行完成但证据不足 | 不能提升为 PASS，需补 QoI、数据或模型。 |
| NOT_EVALUATED | 执行失败或取消 | 不能评价任务目标。 |

禁止规则：不得删除 QoI 后判 PASS；不得放宽门限后判 PASS；不得将 proxy 声明为 native；不得把 delivery 成功等同于 effect 成功；不得把 mission FAIL 自动修复为 PASS。

# 4. 测试矩阵分布

| Level | 数量 |
| --- | --- |
| agent | 6 |
| calibration | 4 |
| component | 392 |
| environment | 2 |
| model_contract | 7 |
| release | 2 |
| static_quality | 6 |
| subsystem | 12 |
| whole_spacecraft_or_agent_release | 6 |

| Scenario Type | 数量 |
| --- | --- |
| all_modes | 6 |
| batch | 24 |
| combined_default_set | 24 |
| compileall | 1 |
| degradation | 48 |
| degradation_default_set | 24 |
| determinism | 24 |
| duplicate_definitions | 1 |
| duration_boundary | 48 |
| exception_policy | 1 |
| export_script | 1 |
| fault | 48 |
| fault_default_set | 24 |
| golden_repeat5 | 1 |
| invalid_ground_calibrated_rejection | 1 |
| layer_schema | 1 |
| native_capability_audit | 1 |
| negative | 24 |
| nominal | 24 |
| normal_working | 80 |
| parameter_provenance | 1 |
| parse_template | 1 |
| plan_whole | 1 |
| public_exports | 1 |
| public_type_hints | 1 |
| pytest | 1 |
| readiness_check | 1 |
| readiness_report | 1 |
| release_check | 1 |
| repeat_stability | 6 |
| representative_run | 6 |
| resolve_component | 1 |
| ruff_blockers | 1 |
| runtime_injection | 1 |
| schema | 1 |
| strict_doctor | 1 |
| subsystem_builders | 1 |
| subsystem_runners | 1 |
| validate_component | 1 |
| version | 1 |
| whole_spacecraft_native | 1 |

# 5. 部件级测试覆盖

| 部件 | 测试项数量 | 覆盖场景类型 |
| --- | --- | --- |
| antenna | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| battery | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| cmg | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| data_queue | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| fuel_tank | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| ground_station | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| heater | 13 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1 |
| imu | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| link_budget | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| magnetometer | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| mtb | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| onboard_storage | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| payload | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| payload_sensor | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| pdu | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| power_sink | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| radiator | 13 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1 |
| reaction_wheel | 13 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1 |
| solar_panel | 13 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1 |
| star_tracker | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| sun_sensor | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| thermal_node | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| thruster | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |
| transmitter | 17 | batch:1, combined_default_set:1, degradation:2, degradation_default_set:1, determinism:1, duration_boundary:2, fault:2, fault_default_set:1, negative:1, nominal:1, normal_working:4 |

# 6. 完整测试项明细（437项）

下表列出所有测试项。命令均可直接从工程根目录执行。判读参数采用 JSON 摘要；完整版本见 configs/verification/engineering_verification_matrix.json。

| Test ID | 层级 | 对象 | 场景类型/场景 | 执行命令 | 判读参数 | 通过规则 |
| --- | --- | --- | --- | --- | --- | --- |
| CMP-NOM-ANTENNA-NOMINAL | component | antenna | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-ANTENNA-NOMINAL --output reports//CMP-NOM-ANTENNA-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-ANTENNA-NOMINAL | component | antenna | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-ANTENNA-NOMINAL --output reports//CMP-WRK-ANTENNA-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-ANTENNA-BORESIGHT-TRACKING | component | antenna | normal_working / boresight_tracking | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-ANTENNA-BORESIGHT-TRACKING --output reports//CMP-WRK-ANTENNA-BORESIGHT-TRACKING.json | {"finite_numeric_required":true,"scenario":"boresight_tracking"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-ANTENNA-SLEW-ACQUISITION | component | antenna | normal_working / slew_acquisition | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-ANTENNA-SLEW-ACQUISITION --output reports//CMP-WRK-ANTENNA-SLEW-ACQUISITION.json | {"finite_numeric_required":true,"scenario":"slew_acquisition"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-ANTENNA-HALF-POWER-EDGE | component | antenna | normal_working / half_power_edge | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-ANTENNA-HALF-POWER-EDGE --output reports//CMP-WRK-ANTENNA-HALF-POWER-EDGE.json | {"finite_numeric_required":true,"scenario":"half_power_edge"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-ANTENNA-RUN-ALL-MODES | component | antenna | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-ANTENNA-RUN-ALL-MODES --output reports//CMP-ALL-ANTENNA-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-ANTENNA-POINTING-LOSS | component | antenna | fault / pointing_loss | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-ANTENNA-POINTING-LOSS --output reports//CMP-FLT-ANTENNA-POINTING-LOSS.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-ANTENNA-FEED-OPEN | component | antenna | fault / feed_open | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-ANTENNA-FEED-OPEN --output reports//CMP-FLT-ANTENNA-FEED-OPEN.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-ANTENNA-SURFACE-CONTAMINATION | component | antenna | degradation / surface_contamination | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-ANTENNA-SURFACE-CONTAMINATION --output reports//CMP-DEG-ANTENNA-SURFACE-CONTAMINATION.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-ANTENNA-POINTING-BIAS-DRIFT | component | antenna | degradation / pointing_bias_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-ANTENNA-POINTING-BIAS-DRIFT --output reports//CMP-DEG-ANTENNA-POINTING-BIAS-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-ANTENNA-FAULT-DEFAULT-SET | component | antenna | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-ANTENNA-FAULT-DEFAULT-SET --output reports//CMP-L2-ANTENNA-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-ANTENNA-DEGRADATION-DEFAULT-SET | component | antenna | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-ANTENNA-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-ANTENNA-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-ANTENNA-COMBINED-DEFAULT-SET | component | antenna | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-ANTENNA-COMBINED-DEFAULT-SET --output reports//CMP-L2-ANTENNA-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-ANTENNA-SHORT-DURATION-60S | component | antenna | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-ANTENNA-SHORT-DURATION-60S --output reports//CMP-BND-ANTENNA-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-ANTENNA-LONG-DURATION-86400S | component | antenna | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-ANTENNA-LONG-DURATION-86400S --output reports//CMP-BND-ANTENNA-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-ANTENNA-UNSUPPORTED-WORKING-SCENARIO | component | antenna | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-ANTENNA-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-ANTENNA-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-ANTENNA-REPEAT-STABILITY | component | antenna | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-ANTENNA-REPEAT-STABILITY --output reports//CMP-DET-ANTENNA-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-BATTERY-NOMINAL | component | battery | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-BATTERY-NOMINAL --output reports//CMP-NOM-BATTERY-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-BATTERY-NOMINAL | component | battery | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-BATTERY-NOMINAL --output reports//CMP-WRK-BATTERY-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-BATTERY-CHARGE-WINDOW | component | battery | normal_working / charge_window | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-BATTERY-CHARGE-WINDOW --output reports//CMP-WRK-BATTERY-CHARGE-WINDOW.json | {"finite_numeric_required":true,"scenario":"charge_window"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-BATTERY-ECLIPSE-DISCHARGE | component | battery | normal_working / eclipse_discharge | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-BATTERY-ECLIPSE-DISCHARGE --output reports//CMP-WRK-BATTERY-ECLIPSE-DISCHARGE.json | {"finite_numeric_required":true,"scenario":"eclipse_discharge"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-BATTERY-ORBIT-CYCLE | component | battery | normal_working / orbit_cycle | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-BATTERY-ORBIT-CYCLE --output reports//CMP-WRK-BATTERY-ORBIT-CYCLE.json | {"finite_numeric_required":true,"scenario":"orbit_cycle"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-BATTERY-RUN-ALL-MODES | component | battery | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-BATTERY-RUN-ALL-MODES --output reports//CMP-ALL-BATTERY-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-BATTERY-OPEN-CIRCUIT | component | battery | fault / open_circuit | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-BATTERY-OPEN-CIRCUIT --output reports//CMP-FLT-BATTERY-OPEN-CIRCUIT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-BATTERY-INTERNAL-SHORT | component | battery | fault / internal_short | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-BATTERY-INTERNAL-SHORT --output reports//CMP-FLT-BATTERY-INTERNAL-SHORT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-BATTERY-CYCLE-CAPACITY-FADE | component | battery | degradation / cycle_capacity_fade | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-BATTERY-CYCLE-CAPACITY-FADE --output reports//CMP-DEG-BATTERY-CYCLE-CAPACITY-FADE.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-BATTERY-RESISTANCE-GROWTH | component | battery | degradation / resistance_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-BATTERY-RESISTANCE-GROWTH --output reports//CMP-DEG-BATTERY-RESISTANCE-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-BATTERY-FAULT-DEFAULT-SET | component | battery | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-BATTERY-FAULT-DEFAULT-SET --output reports//CMP-L2-BATTERY-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-BATTERY-DEGRADATION-DEFAULT-SET | component | battery | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-BATTERY-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-BATTERY-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-BATTERY-COMBINED-DEFAULT-SET | component | battery | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-BATTERY-COMBINED-DEFAULT-SET --output reports//CMP-L2-BATTERY-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-BATTERY-SHORT-DURATION-60S | component | battery | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-BATTERY-SHORT-DURATION-60S --output reports//CMP-BND-BATTERY-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-BATTERY-LONG-DURATION-86400S | component | battery | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-BATTERY-LONG-DURATION-86400S --output reports//CMP-BND-BATTERY-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-BATTERY-UNSUPPORTED-WORKING-SCENARIO | component | battery | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-BATTERY-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-BATTERY-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-BATTERY-REPEAT-STABILITY | component | battery | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-BATTERY-REPEAT-STABILITY --output reports//CMP-DET-BATTERY-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-CMG-NOMINAL | component | cmg | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-CMG-NOMINAL --output reports//CMP-NOM-CMG-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-CMG-NOMINAL | component | cmg | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-CMG-NOMINAL --output reports//CMP-WRK-CMG-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-CMG-SMALL-SLEW | component | cmg | normal_working / small_slew | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-CMG-SMALL-SLEW --output reports//CMP-WRK-CMG-SMALL-SLEW.json | {"finite_numeric_required":true,"scenario":"small_slew"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-CMG-SLEW-REVERSAL | component | cmg | normal_working / slew_reversal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-CMG-SLEW-REVERSAL --output reports//CMP-WRK-CMG-SLEW-REVERSAL.json | {"finite_numeric_required":true,"scenario":"slew_reversal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-CMG-WHEEL-SPEED-TRIM | component | cmg | normal_working / wheel_speed_trim | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-CMG-WHEEL-SPEED-TRIM --output reports//CMP-WRK-CMG-WHEEL-SPEED-TRIM.json | {"finite_numeric_required":true,"scenario":"wheel_speed_trim"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-CMG-RUN-ALL-MODES | component | cmg | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-CMG-RUN-ALL-MODES --output reports//CMP-ALL-CMG-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-CMG-GIMBAL-STUCK | component | cmg | fault / gimbal_stuck | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-CMG-GIMBAL-STUCK --output reports//CMP-FLT-CMG-GIMBAL-STUCK.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-CMG-WHEEL-DRIVE-FAULT | component | cmg | fault / wheel_drive_fault | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-CMG-WHEEL-DRIVE-FAULT --output reports//CMP-FLT-CMG-WHEEL-DRIVE-FAULT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-CMG-GIMBAL-FRICTION-GROWTH | component | cmg | degradation / gimbal_friction_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-CMG-GIMBAL-FRICTION-GROWTH --output reports//CMP-DEG-CMG-GIMBAL-FRICTION-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-CMG-MOMENTUM-LOSS | component | cmg | degradation / momentum_loss | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-CMG-MOMENTUM-LOSS --output reports//CMP-DEG-CMG-MOMENTUM-LOSS.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-CMG-FAULT-DEFAULT-SET | component | cmg | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-CMG-FAULT-DEFAULT-SET --output reports//CMP-L2-CMG-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-CMG-DEGRADATION-DEFAULT-SET | component | cmg | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-CMG-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-CMG-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-CMG-COMBINED-DEFAULT-SET | component | cmg | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-CMG-COMBINED-DEFAULT-SET --output reports//CMP-L2-CMG-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-CMG-SHORT-DURATION-60S | component | cmg | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-CMG-SHORT-DURATION-60S --output reports//CMP-BND-CMG-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-CMG-LONG-DURATION-86400S | component | cmg | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-CMG-LONG-DURATION-86400S --output reports//CMP-BND-CMG-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-CMG-UNSUPPORTED-WORKING-SCENARIO | component | cmg | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-CMG-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-CMG-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-CMG-REPEAT-STABILITY | component | cmg | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-CMG-REPEAT-STABILITY --output reports//CMP-DET-CMG-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-DATA-QUEUE-NOMINAL | component | data_queue | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-DATA-QUEUE-NOMINAL --output reports//CMP-NOM-DATA-QUEUE-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-DATA-QUEUE-NOMINAL | component | data_queue | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-DATA-QUEUE-NOMINAL --output reports//CMP-WRK-DATA-QUEUE-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-DATA-QUEUE-IMAGING-ACCUMULATION | component | data_queue | normal_working / imaging_accumulation | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-DATA-QUEUE-IMAGING-ACCUMULATION --output reports//CMP-WRK-DATA-QUEUE-IMAGING-ACCUMULATION.json | {"finite_numeric_required":true,"scenario":"imaging_accumulation"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-DATA-QUEUE-DOWNLINK-PASS | component | data_queue | normal_working / downlink_pass | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-DATA-QUEUE-DOWNLINK-PASS --output reports//CMP-WRK-DATA-QUEUE-DOWNLINK-PASS.json | {"finite_numeric_required":true,"scenario":"downlink_pass"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-DATA-QUEUE-OVERFLOW-PROTECTION | component | data_queue | normal_working / overflow_protection | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-DATA-QUEUE-OVERFLOW-PROTECTION --output reports//CMP-WRK-DATA-QUEUE-OVERFLOW-PROTECTION.json | {"finite_numeric_required":true,"scenario":"overflow_protection"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-DATA-QUEUE-RUN-ALL-MODES | component | data_queue | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-DATA-QUEUE-RUN-ALL-MODES --output reports//CMP-ALL-DATA-QUEUE-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-DATA-QUEUE-OVERFLOW | component | data_queue | fault / overflow | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-DATA-QUEUE-OVERFLOW --output reports//CMP-FLT-DATA-QUEUE-OVERFLOW.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-DATA-QUEUE-READ-STALL | component | data_queue | fault / read_stall | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-DATA-QUEUE-READ-STALL --output reports//CMP-FLT-DATA-QUEUE-READ-STALL.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-DATA-QUEUE-THROUGHPUT-DECAY | component | data_queue | degradation / throughput_decay | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-DATA-QUEUE-THROUGHPUT-DECAY --output reports//CMP-DEG-DATA-QUEUE-THROUGHPUT-DECAY.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-DATA-QUEUE-PACKET-ERROR-GROWTH | component | data_queue | degradation / packet_error_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-DATA-QUEUE-PACKET-ERROR-GROWTH --output reports//CMP-DEG-DATA-QUEUE-PACKET-ERROR-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-DATA-QUEUE-FAULT-DEFAULT-SET | component | data_queue | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-DATA-QUEUE-FAULT-DEFAULT-SET --output reports//CMP-L2-DATA-QUEUE-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-DATA-QUEUE-DEGRADATION-DEFAULT-SET | component | data_queue | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-DATA-QUEUE-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-DATA-QUEUE-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-DATA-QUEUE-COMBINED-DEFAULT-SET | component | data_queue | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-DATA-QUEUE-COMBINED-DEFAULT-SET --output reports//CMP-L2-DATA-QUEUE-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-DATA-QUEUE-SHORT-DURATION-60S | component | data_queue | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-DATA-QUEUE-SHORT-DURATION-60S --output reports//CMP-BND-DATA-QUEUE-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-DATA-QUEUE-LONG-DURATION-86400S | component | data_queue | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-DATA-QUEUE-LONG-DURATION-86400S --output reports//CMP-BND-DATA-QUEUE-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-DATA-QUEUE-UNSUPPORTED-WORKING-SCENARIO | component | data_queue | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-DATA-QUEUE-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-DATA-QUEUE-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-DATA-QUEUE-REPEAT-STABILITY | component | data_queue | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-DATA-QUEUE-REPEAT-STABILITY --output reports//CMP-DET-DATA-QUEUE-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-FUEL-TANK-NOMINAL | component | fuel_tank | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-FUEL-TANK-NOMINAL --output reports//CMP-NOM-FUEL-TANK-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-FUEL-TANK-NOMINAL | component | fuel_tank | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-FUEL-TANK-NOMINAL --output reports//CMP-WRK-FUEL-TANK-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-FUEL-TANK-STANDBY-NO-FLOW | component | fuel_tank | normal_working / standby_no_flow | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-FUEL-TANK-STANDBY-NO-FLOW --output reports//CMP-WRK-FUEL-TANK-STANDBY-NO-FLOW.json | {"finite_numeric_required":true,"scenario":"standby_no_flow"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-FUEL-TANK-STATION-KEEPING-BURN | component | fuel_tank | normal_working / station_keeping_burn | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-FUEL-TANK-STATION-KEEPING-BURN --output reports//CMP-WRK-FUEL-TANK-STATION-KEEPING-BURN.json | {"finite_numeric_required":true,"scenario":"station_keeping_burn"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-FUEL-TANK-HIGH-RATE-BURN | component | fuel_tank | normal_working / high_rate_burn | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-FUEL-TANK-HIGH-RATE-BURN --output reports//CMP-WRK-FUEL-TANK-HIGH-RATE-BURN.json | {"finite_numeric_required":true,"scenario":"high_rate_burn"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-FUEL-TANK-RUN-ALL-MODES | component | fuel_tank | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-FUEL-TANK-RUN-ALL-MODES --output reports//CMP-ALL-FUEL-TANK-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-FUEL-TANK-LEAK | component | fuel_tank | fault / leak | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-FUEL-TANK-LEAK --output reports//CMP-FLT-FUEL-TANK-LEAK.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-FUEL-TANK-OUTLET-BLOCKAGE | component | fuel_tank | fault / outlet_blockage | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-FUEL-TANK-OUTLET-BLOCKAGE --output reports//CMP-FLT-FUEL-TANK-OUTLET-BLOCKAGE.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-FUEL-TANK-PRESSURE-DECAY | component | fuel_tank | degradation / pressure_decay | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-FUEL-TANK-PRESSURE-DECAY --output reports//CMP-DEG-FUEL-TANK-PRESSURE-DECAY.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-FUEL-TANK-OUTLET-AGING | component | fuel_tank | degradation / outlet_aging | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-FUEL-TANK-OUTLET-AGING --output reports//CMP-DEG-FUEL-TANK-OUTLET-AGING.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-FUEL-TANK-FAULT-DEFAULT-SET | component | fuel_tank | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-FUEL-TANK-FAULT-DEFAULT-SET --output reports//CMP-L2-FUEL-TANK-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-FUEL-TANK-DEGRADATION-DEFAULT-SET | component | fuel_tank | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-FUEL-TANK-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-FUEL-TANK-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-FUEL-TANK-COMBINED-DEFAULT-SET | component | fuel_tank | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-FUEL-TANK-COMBINED-DEFAULT-SET --output reports//CMP-L2-FUEL-TANK-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-FUEL-TANK-SHORT-DURATION-60S | component | fuel_tank | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-FUEL-TANK-SHORT-DURATION-60S --output reports//CMP-BND-FUEL-TANK-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-FUEL-TANK-LONG-DURATION-86400S | component | fuel_tank | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-FUEL-TANK-LONG-DURATION-86400S --output reports//CMP-BND-FUEL-TANK-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-FUEL-TANK-UNSUPPORTED-WORKING-SCENARIO | component | fuel_tank | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-FUEL-TANK-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-FUEL-TANK-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-FUEL-TANK-REPEAT-STABILITY | component | fuel_tank | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-FUEL-TANK-REPEAT-STABILITY --output reports//CMP-DET-FUEL-TANK-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-GROUND-STATION-NOMINAL | component | ground_station | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-GROUND-STATION-NOMINAL --output reports//CMP-NOM-GROUND-STATION-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-GROUND-STATION-NOMINAL | component | ground_station | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-GROUND-STATION-NOMINAL --output reports//CMP-WRK-GROUND-STATION-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-GROUND-STATION-OVERHEAD-PASS | component | ground_station | normal_working / overhead_pass | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-GROUND-STATION-OVERHEAD-PASS --output reports//CMP-WRK-GROUND-STATION-OVERHEAD-PASS.json | {"finite_numeric_required":true,"scenario":"overhead_pass"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-GROUND-STATION-HORIZON-MISS | component | ground_station | normal_working / horizon_miss | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-GROUND-STATION-HORIZON-MISS --output reports//CMP-WRK-GROUND-STATION-HORIZON-MISS.json | {"finite_numeric_required":true,"scenario":"horizon_miss"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-GROUND-STATION-STRICT-ELEVATION-MASK | component | ground_station | normal_working / strict_elevation_mask | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-GROUND-STATION-STRICT-ELEVATION-MASK --output reports//CMP-WRK-GROUND-STATION-STRICT-ELEVATION-MASK.json | {"finite_numeric_required":true,"scenario":"strict_elevation_mask"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-GROUND-STATION-RUN-ALL-MODES | component | ground_station | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-GROUND-STATION-RUN-ALL-MODES --output reports//CMP-ALL-GROUND-STATION-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-GROUND-STATION-TRACKING-LOSS | component | ground_station | fault / tracking_loss | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-GROUND-STATION-TRACKING-LOSS --output reports//CMP-FLT-GROUND-STATION-TRACKING-LOSS.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-GROUND-STATION-WEATHER-FADE | component | ground_station | fault / weather_fade | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-GROUND-STATION-WEATHER-FADE --output reports//CMP-FLT-GROUND-STATION-WEATHER-FADE.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-GROUND-STATION-GOVER-TDRIFT | component | ground_station | degradation / gover_tdrift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-GROUND-STATION-GOVER-TDRIFT --output reports//CMP-DEG-GROUND-STATION-GOVER-TDRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-GROUND-STATION-POINTING-ACCURACY-DRIFT | component | ground_station | degradation / pointing_accuracy_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-GROUND-STATION-POINTING-ACCURACY-DRIFT --output reports//CMP-DEG-GROUND-STATION-POINTING-ACCURACY-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-GROUND-STATION-FAULT-DEFAULT-SET | component | ground_station | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-GROUND-STATION-FAULT-DEFAULT-SET --output reports//CMP-L2-GROUND-STATION-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-GROUND-STATION-DEGRADATION-DEFAULT-SET | component | ground_station | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-GROUND-STATION-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-GROUND-STATION-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-GROUND-STATION-COMBINED-DEFAULT-SET | component | ground_station | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-GROUND-STATION-COMBINED-DEFAULT-SET --output reports//CMP-L2-GROUND-STATION-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-GROUND-STATION-SHORT-DURATION-60S | component | ground_station | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-GROUND-STATION-SHORT-DURATION-60S --output reports//CMP-BND-GROUND-STATION-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-GROUND-STATION-LONG-DURATION-86400S | component | ground_station | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-GROUND-STATION-LONG-DURATION-86400S --output reports//CMP-BND-GROUND-STATION-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-GROUND-STATION-UNSUPPORTED-WORKING-SCENARIO | component | ground_station | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-GROUND-STATION-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-GROUND-STATION-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-GROUND-STATION-REPEAT-STABILITY | component | ground_station | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-GROUND-STATION-REPEAT-STABILITY --output reports//CMP-DET-GROUND-STATION-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-HEATER-NOMINAL | component | heater | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-HEATER-NOMINAL --output reports//CMP-NOM-HEATER-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-ALL-HEATER-RUN-ALL-MODES | component | heater | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-HEATER-RUN-ALL-MODES --output reports//CMP-ALL-HEATER-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-HEATER-STUCK-OFF | component | heater | fault / stuck_off | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-HEATER-STUCK-OFF --output reports//CMP-FLT-HEATER-STUCK-OFF.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-HEATER-STUCK-ON | component | heater | fault / stuck_on | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-HEATER-STUCK-ON --output reports//CMP-FLT-HEATER-STUCK-ON.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-HEATER-HEATING-EFFICIENCY-DECAY | component | heater | degradation / heating_efficiency_decay | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-HEATER-HEATING-EFFICIENCY-DECAY --output reports//CMP-DEG-HEATER-HEATING-EFFICIENCY-DECAY.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-HEATER-RESISTANCE-DRIFT | component | heater | degradation / resistance_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-HEATER-RESISTANCE-DRIFT --output reports//CMP-DEG-HEATER-RESISTANCE-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-HEATER-FAULT-DEFAULT-SET | component | heater | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-HEATER-FAULT-DEFAULT-SET --output reports//CMP-L2-HEATER-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-HEATER-DEGRADATION-DEFAULT-SET | component | heater | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-HEATER-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-HEATER-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-HEATER-COMBINED-DEFAULT-SET | component | heater | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-HEATER-COMBINED-DEFAULT-SET --output reports//CMP-L2-HEATER-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-HEATER-SHORT-DURATION-60S | component | heater | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-HEATER-SHORT-DURATION-60S --output reports//CMP-BND-HEATER-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-HEATER-LONG-DURATION-86400S | component | heater | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-HEATER-LONG-DURATION-86400S --output reports//CMP-BND-HEATER-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-HEATER-UNSUPPORTED-WORKING-SCENARIO | component | heater | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-HEATER-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-HEATER-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-HEATER-REPEAT-STABILITY | component | heater | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-HEATER-REPEAT-STABILITY --output reports//CMP-DET-HEATER-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-IMU-NOMINAL | component | imu | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-IMU-NOMINAL --output reports//CMP-NOM-IMU-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-IMU-NOMINAL | component | imu | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-IMU-NOMINAL --output reports//CMP-WRK-IMU-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-IMU-SHORT-BIAS-WALK | component | imu | normal_working / short_bias_walk | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-IMU-SHORT-BIAS-WALK --output reports//CMP-WRK-IMU-SHORT-BIAS-WALK.json | {"finite_numeric_required":true,"scenario":"short_bias_walk"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-IMU-LONG-BIAS-WALK | component | imu | normal_working / long_bias_walk | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-IMU-LONG-BIAS-WALK --output reports//CMP-WRK-IMU-LONG-BIAS-WALK.json | {"finite_numeric_required":true,"scenario":"long_bias_walk"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-IMU-MEASUREMENT-READOUT | component | imu | normal_working / measurement_readout | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-IMU-MEASUREMENT-READOUT --output reports//CMP-WRK-IMU-MEASUREMENT-READOUT.json | {"finite_numeric_required":true,"scenario":"measurement_readout"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-IMU-RUN-ALL-MODES | component | imu | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-IMU-RUN-ALL-MODES --output reports//CMP-ALL-IMU-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-IMU-BIAS-STEP | component | imu | fault / bias_step | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-IMU-BIAS-STEP --output reports//CMP-FLT-IMU-BIAS-STEP.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-IMU-AXIS-DROPOUT | component | imu | fault / axis_dropout | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-IMU-AXIS-DROPOUT --output reports//CMP-FLT-IMU-AXIS-DROPOUT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-IMU-BIAS-DRIFT | component | imu | degradation / bias_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-IMU-BIAS-DRIFT --output reports//CMP-DEG-IMU-BIAS-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-IMU-NOISE-DENSITY-GROWTH | component | imu | degradation / noise_density_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-IMU-NOISE-DENSITY-GROWTH --output reports//CMP-DEG-IMU-NOISE-DENSITY-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-IMU-FAULT-DEFAULT-SET | component | imu | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-IMU-FAULT-DEFAULT-SET --output reports//CMP-L2-IMU-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-IMU-DEGRADATION-DEFAULT-SET | component | imu | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-IMU-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-IMU-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-IMU-COMBINED-DEFAULT-SET | component | imu | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-IMU-COMBINED-DEFAULT-SET --output reports//CMP-L2-IMU-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-IMU-SHORT-DURATION-60S | component | imu | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-IMU-SHORT-DURATION-60S --output reports//CMP-BND-IMU-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-IMU-LONG-DURATION-86400S | component | imu | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-IMU-LONG-DURATION-86400S --output reports//CMP-BND-IMU-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-IMU-UNSUPPORTED-WORKING-SCENARIO | component | imu | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-IMU-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-IMU-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-IMU-REPEAT-STABILITY | component | imu | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-IMU-REPEAT-STABILITY --output reports//CMP-DET-IMU-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-LINK-BUDGET-NOMINAL | component | link_budget | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-LINK-BUDGET-NOMINAL --output reports//CMP-NOM-LINK-BUDGET-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-LINK-BUDGET-NOMINAL | component | link_budget | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-LINK-BUDGET-NOMINAL --output reports//CMP-WRK-LINK-BUDGET-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-LINK-BUDGET-NEAR-CONTACT | component | link_budget | normal_working / near_contact | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-LINK-BUDGET-NEAR-CONTACT --output reports//CMP-WRK-LINK-BUDGET-NEAR-CONTACT.json | {"finite_numeric_required":true,"scenario":"near_contact"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-LINK-BUDGET-ORBIT-RANGE-SWEEP | component | link_budget | normal_working / orbit_range_sweep | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-LINK-BUDGET-ORBIT-RANGE-SWEEP --output reports//CMP-WRK-LINK-BUDGET-ORBIT-RANGE-SWEEP.json | {"finite_numeric_required":true,"scenario":"orbit_range_sweep"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-LINK-BUDGET-LOW-POWER-DOWNLINK | component | link_budget | normal_working / low_power_downlink | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-LINK-BUDGET-LOW-POWER-DOWNLINK --output reports//CMP-WRK-LINK-BUDGET-LOW-POWER-DOWNLINK.json | {"finite_numeric_required":true,"scenario":"low_power_downlink"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-LINK-BUDGET-RUN-ALL-MODES | component | link_budget | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-LINK-BUDGET-RUN-ALL-MODES --output reports//CMP-ALL-LINK-BUDGET-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-LINK-BUDGET-JAMMING | component | link_budget | fault / jamming | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-LINK-BUDGET-JAMMING --output reports//CMP-FLT-LINK-BUDGET-JAMMING.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-LINK-BUDGET-FADE | component | link_budget | fault / fade | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-LINK-BUDGET-FADE --output reports//CMP-FLT-LINK-BUDGET-FADE.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-LINK-BUDGET-MARGIN-EROSION | component | link_budget | degradation / margin_erosion | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-LINK-BUDGET-MARGIN-EROSION --output reports//CMP-DEG-LINK-BUDGET-MARGIN-EROSION.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-LINK-BUDGET-NOISE-FIGURE-GROWTH | component | link_budget | degradation / noise_figure_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-LINK-BUDGET-NOISE-FIGURE-GROWTH --output reports//CMP-DEG-LINK-BUDGET-NOISE-FIGURE-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-LINK-BUDGET-FAULT-DEFAULT-SET | component | link_budget | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-LINK-BUDGET-FAULT-DEFAULT-SET --output reports//CMP-L2-LINK-BUDGET-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-LINK-BUDGET-DEGRADATION-DEFAULT-SET | component | link_budget | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-LINK-BUDGET-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-LINK-BUDGET-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-LINK-BUDGET-COMBINED-DEFAULT-SET | component | link_budget | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-LINK-BUDGET-COMBINED-DEFAULT-SET --output reports//CMP-L2-LINK-BUDGET-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-LINK-BUDGET-SHORT-DURATION-60S | component | link_budget | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-LINK-BUDGET-SHORT-DURATION-60S --output reports//CMP-BND-LINK-BUDGET-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-LINK-BUDGET-LONG-DURATION-86400S | component | link_budget | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-LINK-BUDGET-LONG-DURATION-86400S --output reports//CMP-BND-LINK-BUDGET-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-LINK-BUDGET-UNSUPPORTED-WORKING-SCENARIO | component | link_budget | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-LINK-BUDGET-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-LINK-BUDGET-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-LINK-BUDGET-REPEAT-STABILITY | component | link_budget | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-LINK-BUDGET-REPEAT-STABILITY --output reports//CMP-DET-LINK-BUDGET-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-MAGNETOMETER-NOMINAL | component | magnetometer | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-MAGNETOMETER-NOMINAL --output reports//CMP-NOM-MAGNETOMETER-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-MAGNETOMETER-NOMINAL | component | magnetometer | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-MAGNETOMETER-NOMINAL --output reports//CMP-WRK-MAGNETOMETER-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-MAGNETOMETER-BODY-FRAME-MEASUREMENT | component | magnetometer | normal_working / body_frame_measurement | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-MAGNETOMETER-BODY-FRAME-MEASUREMENT --output reports//CMP-WRK-MAGNETOMETER-BODY-FRAME-MEASUREMENT.json | {"finite_numeric_required":true,"scenario":"body_frame_measurement"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-MAGNETOMETER-ROTATED-MOUNTING | component | magnetometer | normal_working / rotated_mounting | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-MAGNETOMETER-ROTATED-MOUNTING --output reports//CMP-WRK-MAGNETOMETER-ROTATED-MOUNTING.json | {"finite_numeric_required":true,"scenario":"rotated_mounting"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-MAGNETOMETER-BIASED-READOUT | component | magnetometer | normal_working / biased_readout | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-MAGNETOMETER-BIASED-READOUT --output reports//CMP-WRK-MAGNETOMETER-BIASED-READOUT.json | {"finite_numeric_required":true,"scenario":"biased_readout"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-MAGNETOMETER-RUN-ALL-MODES | component | magnetometer | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-MAGNETOMETER-RUN-ALL-MODES --output reports//CMP-ALL-MAGNETOMETER-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-MAGNETOMETER-AXIS-BIAS | component | magnetometer | fault / axis_bias | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-MAGNETOMETER-AXIS-BIAS --output reports//CMP-FLT-MAGNETOMETER-AXIS-BIAS.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-MAGNETOMETER-AXIS-DROPOUT | component | magnetometer | fault / axis_dropout | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-MAGNETOMETER-AXIS-DROPOUT --output reports//CMP-FLT-MAGNETOMETER-AXIS-DROPOUT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-MAGNETOMETER-BIAS-DRIFT | component | magnetometer | degradation / bias_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-MAGNETOMETER-BIAS-DRIFT --output reports//CMP-DEG-MAGNETOMETER-BIAS-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-MAGNETOMETER-SCALE-FACTOR-DRIFT | component | magnetometer | degradation / scale_factor_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-MAGNETOMETER-SCALE-FACTOR-DRIFT --output reports//CMP-DEG-MAGNETOMETER-SCALE-FACTOR-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-MAGNETOMETER-FAULT-DEFAULT-SET | component | magnetometer | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-MAGNETOMETER-FAULT-DEFAULT-SET --output reports//CMP-L2-MAGNETOMETER-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-MAGNETOMETER-DEGRADATION-DEFAULT-SET | component | magnetometer | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-MAGNETOMETER-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-MAGNETOMETER-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-MAGNETOMETER-COMBINED-DEFAULT-SET | component | magnetometer | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-MAGNETOMETER-COMBINED-DEFAULT-SET --output reports//CMP-L2-MAGNETOMETER-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-MAGNETOMETER-SHORT-DURATION-60S | component | magnetometer | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-MAGNETOMETER-SHORT-DURATION-60S --output reports//CMP-BND-MAGNETOMETER-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-MAGNETOMETER-LONG-DURATION-86400S | component | magnetometer | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-MAGNETOMETER-LONG-DURATION-86400S --output reports//CMP-BND-MAGNETOMETER-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-MAGNETOMETER-UNSUPPORTED-WORKING-SCENARIO | component | magnetometer | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-MAGNETOMETER-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-MAGNETOMETER-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-MAGNETOMETER-REPEAT-STABILITY | component | magnetometer | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-MAGNETOMETER-REPEAT-STABILITY --output reports//CMP-DET-MAGNETOMETER-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-MTB-NOMINAL | component | mtb | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-MTB-NOMINAL --output reports//CMP-NOM-MTB-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-MTB-NOMINAL | component | mtb | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-MTB-NOMINAL --output reports//CMP-WRK-MTB-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-MTB-SINGLE-AXIS-DIPOLE | component | mtb | normal_working / single_axis_dipole | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-MTB-SINGLE-AXIS-DIPOLE --output reports//CMP-WRK-MTB-SINGLE-AXIS-DIPOLE.json | {"finite_numeric_required":true,"scenario":"single_axis_dipole"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-MTB-AXIS-REVERSAL | component | mtb | normal_working / axis_reversal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-MTB-AXIS-REVERSAL --output reports//CMP-WRK-MTB-AXIS-REVERSAL.json | {"finite_numeric_required":true,"scenario":"axis_reversal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-MTB-SATURATED-COMMAND | component | mtb | normal_working / saturated_command | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-MTB-SATURATED-COMMAND --output reports//CMP-WRK-MTB-SATURATED-COMMAND.json | {"finite_numeric_required":true,"scenario":"saturated_command"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-MTB-RUN-ALL-MODES | component | mtb | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-MTB-RUN-ALL-MODES --output reports//CMP-ALL-MTB-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-MTB-COIL-OPEN | component | mtb | fault / coil_open | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-MTB-COIL-OPEN --output reports//CMP-FLT-MTB-COIL-OPEN.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-MTB-COIL-SHORT | component | mtb | fault / coil_short | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-MTB-COIL-SHORT --output reports//CMP-FLT-MTB-COIL-SHORT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-MTB-COIL-RESISTANCE-GROWTH | component | mtb | degradation / coil_resistance_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-MTB-COIL-RESISTANCE-GROWTH --output reports//CMP-DEG-MTB-COIL-RESISTANCE-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-MTB-MAGNETIC-MOMENT-DRIFT | component | mtb | degradation / magnetic_moment_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-MTB-MAGNETIC-MOMENT-DRIFT --output reports//CMP-DEG-MTB-MAGNETIC-MOMENT-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-MTB-FAULT-DEFAULT-SET | component | mtb | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-MTB-FAULT-DEFAULT-SET --output reports//CMP-L2-MTB-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-MTB-DEGRADATION-DEFAULT-SET | component | mtb | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-MTB-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-MTB-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-MTB-COMBINED-DEFAULT-SET | component | mtb | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-MTB-COMBINED-DEFAULT-SET --output reports//CMP-L2-MTB-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-MTB-SHORT-DURATION-60S | component | mtb | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-MTB-SHORT-DURATION-60S --output reports//CMP-BND-MTB-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-MTB-LONG-DURATION-86400S | component | mtb | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-MTB-LONG-DURATION-86400S --output reports//CMP-BND-MTB-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-MTB-UNSUPPORTED-WORKING-SCENARIO | component | mtb | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-MTB-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-MTB-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-MTB-REPEAT-STABILITY | component | mtb | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-MTB-REPEAT-STABILITY --output reports//CMP-DET-MTB-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-ONBOARD-STORAGE-NOMINAL | component | onboard_storage | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-ONBOARD-STORAGE-NOMINAL --output reports//CMP-NOM-ONBOARD-STORAGE-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-ONBOARD-STORAGE-NOMINAL | component | onboard_storage | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-ONBOARD-STORAGE-NOMINAL --output reports//CMP-WRK-ONBOARD-STORAGE-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-ONBOARD-STORAGE-PAYLOAD-RECORDING | component | onboard_storage | normal_working / payload_recording | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-ONBOARD-STORAGE-PAYLOAD-RECORDING --output reports//CMP-WRK-ONBOARD-STORAGE-PAYLOAD-RECORDING.json | {"finite_numeric_required":true,"scenario":"payload_recording"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-ONBOARD-STORAGE-DOWNLINK-DRAIN | component | onboard_storage | normal_working / downlink_drain | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-ONBOARD-STORAGE-DOWNLINK-DRAIN --output reports//CMP-WRK-ONBOARD-STORAGE-DOWNLINK-DRAIN.json | {"finite_numeric_required":true,"scenario":"downlink_drain"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-ONBOARD-STORAGE-STORAGE-OVERFLOW | component | onboard_storage | normal_working / storage_overflow | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-ONBOARD-STORAGE-STORAGE-OVERFLOW --output reports//CMP-WRK-ONBOARD-STORAGE-STORAGE-OVERFLOW.json | {"finite_numeric_required":true,"scenario":"storage_overflow"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-ONBOARD-STORAGE-RUN-ALL-MODES | component | onboard_storage | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-ONBOARD-STORAGE-RUN-ALL-MODES --output reports//CMP-ALL-ONBOARD-STORAGE-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-ONBOARD-STORAGE-BAD-BLOCK | component | onboard_storage | fault / bad_block | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-ONBOARD-STORAGE-BAD-BLOCK --output reports//CMP-FLT-ONBOARD-STORAGE-BAD-BLOCK.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-ONBOARD-STORAGE-READ-ONLY | component | onboard_storage | fault / read_only | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-ONBOARD-STORAGE-READ-ONLY --output reports//CMP-FLT-ONBOARD-STORAGE-READ-ONLY.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-ONBOARD-STORAGE-CAPACITY-LOSS | component | onboard_storage | degradation / capacity_loss | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-ONBOARD-STORAGE-CAPACITY-LOSS --output reports//CMP-DEG-ONBOARD-STORAGE-CAPACITY-LOSS.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-ONBOARD-STORAGE-BIT-ERROR-GROWTH | component | onboard_storage | degradation / bit_error_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-ONBOARD-STORAGE-BIT-ERROR-GROWTH --output reports//CMP-DEG-ONBOARD-STORAGE-BIT-ERROR-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-ONBOARD-STORAGE-FAULT-DEFAULT-SET | component | onboard_storage | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-ONBOARD-STORAGE-FAULT-DEFAULT-SET --output reports//CMP-L2-ONBOARD-STORAGE-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-ONBOARD-STORAGE-DEGRADATION-DEFAULT-SET | component | onboard_storage | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-ONBOARD-STORAGE-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-ONBOARD-STORAGE-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-ONBOARD-STORAGE-COMBINED-DEFAULT-SET | component | onboard_storage | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-ONBOARD-STORAGE-COMBINED-DEFAULT-SET --output reports//CMP-L2-ONBOARD-STORAGE-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-ONBOARD-STORAGE-SHORT-DURATION-60S | component | onboard_storage | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-ONBOARD-STORAGE-SHORT-DURATION-60S --output reports//CMP-BND-ONBOARD-STORAGE-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-ONBOARD-STORAGE-LONG-DURATION-86400S | component | onboard_storage | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-ONBOARD-STORAGE-LONG-DURATION-86400S --output reports//CMP-BND-ONBOARD-STORAGE-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-ONBOARD-STORAGE-UNSUPPORTED-WORKING-SCENARIO | component | onboard_storage | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-ONBOARD-STORAGE-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-ONBOARD-STORAGE-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-ONBOARD-STORAGE-REPEAT-STABILITY | component | onboard_storage | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-ONBOARD-STORAGE-REPEAT-STABILITY --output reports//CMP-DET-ONBOARD-STORAGE-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-PAYLOAD-NOMINAL | component | payload | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-PAYLOAD-NOMINAL --output reports//CMP-NOM-PAYLOAD-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-PAYLOAD-NOMINAL | component | payload | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PAYLOAD-NOMINAL --output reports//CMP-WRK-PAYLOAD-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-PAYLOAD-OBSERVATION-PASS | component | payload | normal_working / observation_pass | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PAYLOAD-OBSERVATION-PASS --output reports//CMP-WRK-PAYLOAD-OBSERVATION-PASS.json | {"finite_numeric_required":true,"scenario":"observation_pass"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-PAYLOAD-STANDBY-WINDOW | component | payload | normal_working / standby_window | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PAYLOAD-STANDBY-WINDOW --output reports//CMP-WRK-PAYLOAD-STANDBY-WINDOW.json | {"finite_numeric_required":true,"scenario":"standby_window"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-PAYLOAD-POINTING-LIMITED | component | payload | normal_working / pointing_limited | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PAYLOAD-POINTING-LIMITED --output reports//CMP-WRK-PAYLOAD-POINTING-LIMITED.json | {"finite_numeric_required":true,"scenario":"pointing_limited"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-PAYLOAD-RUN-ALL-MODES | component | payload | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-PAYLOAD-RUN-ALL-MODES --output reports//CMP-ALL-PAYLOAD-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-PAYLOAD-INSTRUMENT-OFF | component | payload | fault / instrument_off | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-PAYLOAD-INSTRUMENT-OFF --output reports//CMP-FLT-PAYLOAD-INSTRUMENT-OFF.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-PAYLOAD-SATURATION | component | payload | fault / saturation | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-PAYLOAD-SATURATION --output reports//CMP-FLT-PAYLOAD-SATURATION.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-PAYLOAD-SENSITIVITY-DECAY | component | payload | degradation / sensitivity_decay | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-PAYLOAD-SENSITIVITY-DECAY --output reports//CMP-DEG-PAYLOAD-SENSITIVITY-DECAY.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-PAYLOAD-DARK-CURRENT-GROWTH | component | payload | degradation / dark_current_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-PAYLOAD-DARK-CURRENT-GROWTH --output reports//CMP-DEG-PAYLOAD-DARK-CURRENT-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-PAYLOAD-FAULT-DEFAULT-SET | component | payload | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-PAYLOAD-FAULT-DEFAULT-SET --output reports//CMP-L2-PAYLOAD-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-PAYLOAD-DEGRADATION-DEFAULT-SET | component | payload | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-PAYLOAD-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-PAYLOAD-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-PAYLOAD-COMBINED-DEFAULT-SET | component | payload | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-PAYLOAD-COMBINED-DEFAULT-SET --output reports//CMP-L2-PAYLOAD-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-PAYLOAD-SHORT-DURATION-60S | component | payload | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-PAYLOAD-SHORT-DURATION-60S --output reports//CMP-BND-PAYLOAD-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-PAYLOAD-LONG-DURATION-86400S | component | payload | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-PAYLOAD-LONG-DURATION-86400S --output reports//CMP-BND-PAYLOAD-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-PAYLOAD-UNSUPPORTED-WORKING-SCENARIO | component | payload | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-PAYLOAD-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-PAYLOAD-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-PAYLOAD-REPEAT-STABILITY | component | payload | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-PAYLOAD-REPEAT-STABILITY --output reports//CMP-DET-PAYLOAD-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-PAYLOAD-SENSOR-NOMINAL | component | payload_sensor | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-PAYLOAD-SENSOR-NOMINAL --output reports//CMP-NOM-PAYLOAD-SENSOR-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-PAYLOAD-SENSOR-NOMINAL | component | payload_sensor | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PAYLOAD-SENSOR-NOMINAL --output reports//CMP-WRK-PAYLOAD-SENSOR-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-PAYLOAD-SENSOR-CLEAR-NADIR | component | payload_sensor | normal_working / clear_nadir | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PAYLOAD-SENSOR-CLEAR-NADIR --output reports//CMP-WRK-PAYLOAD-SENSOR-CLEAR-NADIR.json | {"finite_numeric_required":true,"scenario":"clear_nadir"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-PAYLOAD-SENSOR-CLOUDY-SCENE | component | payload_sensor | normal_working / cloudy_scene | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PAYLOAD-SENSOR-CLOUDY-SCENE --output reports//CMP-WRK-PAYLOAD-SENSOR-CLOUDY-SCENE.json | {"finite_numeric_required":true,"scenario":"cloudy_scene"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-PAYLOAD-SENSOR-POINTING-LIMIT | component | payload_sensor | normal_working / pointing_limit | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PAYLOAD-SENSOR-POINTING-LIMIT --output reports//CMP-WRK-PAYLOAD-SENSOR-POINTING-LIMIT.json | {"finite_numeric_required":true,"scenario":"pointing_limit"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-PAYLOAD-SENSOR-RUN-ALL-MODES | component | payload_sensor | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-PAYLOAD-SENSOR-RUN-ALL-MODES --output reports//CMP-ALL-PAYLOAD-SENSOR-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-PAYLOAD-SENSOR-PIXEL-DROPOUT | component | payload_sensor | fault / pixel_dropout | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-PAYLOAD-SENSOR-PIXEL-DROPOUT --output reports//CMP-FLT-PAYLOAD-SENSOR-PIXEL-DROPOUT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-PAYLOAD-SENSOR-BLUR | component | payload_sensor | fault / blur | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-PAYLOAD-SENSOR-BLUR --output reports//CMP-FLT-PAYLOAD-SENSOR-BLUR.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-PAYLOAD-SENSOR-RESPONSIVITY-DECAY | component | payload_sensor | degradation / responsivity_decay | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-PAYLOAD-SENSOR-RESPONSIVITY-DECAY --output reports//CMP-DEG-PAYLOAD-SENSOR-RESPONSIVITY-DECAY.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-PAYLOAD-SENSOR-NOISE-GROWTH | component | payload_sensor | degradation / noise_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-PAYLOAD-SENSOR-NOISE-GROWTH --output reports//CMP-DEG-PAYLOAD-SENSOR-NOISE-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-PAYLOAD-SENSOR-FAULT-DEFAULT-SET | component | payload_sensor | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-PAYLOAD-SENSOR-FAULT-DEFAULT-SET --output reports//CMP-L2-PAYLOAD-SENSOR-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-PAYLOAD-SENSOR-DEGRADATION-DEFAULT-SET | component | payload_sensor | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-PAYLOAD-SENSOR-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-PAYLOAD-SENSOR-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-PAYLOAD-SENSOR-COMBINED-DEFAULT-SET | component | payload_sensor | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-PAYLOAD-SENSOR-COMBINED-DEFAULT-SET --output reports//CMP-L2-PAYLOAD-SENSOR-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-PAYLOAD-SENSOR-SHORT-DURATION-60S | component | payload_sensor | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-PAYLOAD-SENSOR-SHORT-DURATION-60S --output reports//CMP-BND-PAYLOAD-SENSOR-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-PAYLOAD-SENSOR-LONG-DURATION-86400S | component | payload_sensor | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-PAYLOAD-SENSOR-LONG-DURATION-86400S --output reports//CMP-BND-PAYLOAD-SENSOR-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-PAYLOAD-SENSOR-UNSUPPORTED-WORKING-SCENARIO | component | payload_sensor | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-PAYLOAD-SENSOR-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-PAYLOAD-SENSOR-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-PAYLOAD-SENSOR-REPEAT-STABILITY | component | payload_sensor | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-PAYLOAD-SENSOR-REPEAT-STABILITY --output reports//CMP-DET-PAYLOAD-SENSOR-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-PDU-NOMINAL | component | pdu | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-PDU-NOMINAL --output reports//CMP-NOM-PDU-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-PDU-NOMINAL | component | pdu | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PDU-NOMINAL --output reports//CMP-WRK-PDU-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-PDU-SAFE-MODE-LOADS | component | pdu | normal_working / safe_mode_loads | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PDU-SAFE-MODE-LOADS --output reports//CMP-WRK-PDU-SAFE-MODE-LOADS.json | {"finite_numeric_required":true,"scenario":"safe_mode_loads"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-PDU-PAYLOAD-PEAK-LOADS | component | pdu | normal_working / payload_peak_loads | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PDU-PAYLOAD-PEAK-LOADS --output reports//CMP-WRK-PDU-PAYLOAD-PEAK-LOADS.json | {"finite_numeric_required":true,"scenario":"payload_peak_loads"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-PDU-RECOVERY-LOADS | component | pdu | normal_working / recovery_loads | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-PDU-RECOVERY-LOADS --output reports//CMP-WRK-PDU-RECOVERY-LOADS.json | {"finite_numeric_required":true,"scenario":"recovery_loads"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-PDU-RUN-ALL-MODES | component | pdu | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-PDU-RUN-ALL-MODES --output reports//CMP-ALL-PDU-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-PDU-CHANNEL-TRIP | component | pdu | fault / channel_trip | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-PDU-CHANNEL-TRIP --output reports//CMP-FLT-PDU-CHANNEL-TRIP.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-PDU-OVER-CURRENT | component | pdu | fault / over_current | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-PDU-OVER-CURRENT --output reports//CMP-FLT-PDU-OVER-CURRENT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-PDU-CONTACT-RESISTANCE-GROWTH | component | pdu | degradation / contact_resistance_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-PDU-CONTACT-RESISTANCE-GROWTH --output reports//CMP-DEG-PDU-CONTACT-RESISTANCE-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-PDU-VOLTAGE-REGULATION-DRIFT | component | pdu | degradation / voltage_regulation_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-PDU-VOLTAGE-REGULATION-DRIFT --output reports//CMP-DEG-PDU-VOLTAGE-REGULATION-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-PDU-FAULT-DEFAULT-SET | component | pdu | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-PDU-FAULT-DEFAULT-SET --output reports//CMP-L2-PDU-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-PDU-DEGRADATION-DEFAULT-SET | component | pdu | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-PDU-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-PDU-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-PDU-COMBINED-DEFAULT-SET | component | pdu | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-PDU-COMBINED-DEFAULT-SET --output reports//CMP-L2-PDU-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-PDU-SHORT-DURATION-60S | component | pdu | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-PDU-SHORT-DURATION-60S --output reports//CMP-BND-PDU-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-PDU-LONG-DURATION-86400S | component | pdu | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-PDU-LONG-DURATION-86400S --output reports//CMP-BND-PDU-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-PDU-UNSUPPORTED-WORKING-SCENARIO | component | pdu | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-PDU-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-PDU-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-PDU-REPEAT-STABILITY | component | pdu | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-PDU-REPEAT-STABILITY --output reports//CMP-DET-PDU-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-POWER-SINK-NOMINAL | component | power_sink | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-POWER-SINK-NOMINAL --output reports//CMP-NOM-POWER-SINK-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-POWER-SINK-NOMINAL | component | power_sink | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-POWER-SINK-NOMINAL --output reports//CMP-WRK-POWER-SINK-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-POWER-SINK-SAFE-MODE | component | power_sink | normal_working / safe_mode | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-POWER-SINK-SAFE-MODE --output reports//CMP-WRK-POWER-SINK-SAFE-MODE.json | {"finite_numeric_required":true,"scenario":"safe_mode"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-POWER-SINK-NOMINAL-LOAD | component | power_sink | normal_working / nominal_load | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-POWER-SINK-NOMINAL-LOAD --output reports//CMP-WRK-POWER-SINK-NOMINAL-LOAD.json | {"finite_numeric_required":true,"scenario":"nominal_load"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-POWER-SINK-PAYLOAD-PEAK | component | power_sink | normal_working / payload_peak | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-POWER-SINK-PAYLOAD-PEAK --output reports//CMP-WRK-POWER-SINK-PAYLOAD-PEAK.json | {"finite_numeric_required":true,"scenario":"payload_peak"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-POWER-SINK-RUN-ALL-MODES | component | power_sink | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-POWER-SINK-RUN-ALL-MODES --output reports//CMP-ALL-POWER-SINK-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-POWER-SINK-OVERLOAD | component | power_sink | fault / overload | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-POWER-SINK-OVERLOAD --output reports//CMP-FLT-POWER-SINK-OVERLOAD.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-POWER-SINK-INTERMITTENT-LOAD | component | power_sink | fault / intermittent_load | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-POWER-SINK-INTERMITTENT-LOAD --output reports//CMP-FLT-POWER-SINK-INTERMITTENT-LOAD.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-POWER-SINK-LOAD-DRIFT | component | power_sink | degradation / load_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-POWER-SINK-LOAD-DRIFT --output reports//CMP-DEG-POWER-SINK-LOAD-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-POWER-SINK-EFFICIENCY-DECAY | component | power_sink | degradation / efficiency_decay | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-POWER-SINK-EFFICIENCY-DECAY --output reports//CMP-DEG-POWER-SINK-EFFICIENCY-DECAY.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-POWER-SINK-FAULT-DEFAULT-SET | component | power_sink | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-POWER-SINK-FAULT-DEFAULT-SET --output reports//CMP-L2-POWER-SINK-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-POWER-SINK-DEGRADATION-DEFAULT-SET | component | power_sink | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-POWER-SINK-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-POWER-SINK-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-POWER-SINK-COMBINED-DEFAULT-SET | component | power_sink | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-POWER-SINK-COMBINED-DEFAULT-SET --output reports//CMP-L2-POWER-SINK-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-POWER-SINK-SHORT-DURATION-60S | component | power_sink | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-POWER-SINK-SHORT-DURATION-60S --output reports//CMP-BND-POWER-SINK-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-POWER-SINK-LONG-DURATION-86400S | component | power_sink | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-POWER-SINK-LONG-DURATION-86400S --output reports//CMP-BND-POWER-SINK-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-POWER-SINK-UNSUPPORTED-WORKING-SCENARIO | component | power_sink | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-POWER-SINK-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-POWER-SINK-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-POWER-SINK-REPEAT-STABILITY | component | power_sink | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-POWER-SINK-REPEAT-STABILITY --output reports//CMP-DET-POWER-SINK-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-RADIATOR-NOMINAL | component | radiator | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-RADIATOR-NOMINAL --output reports//CMP-NOM-RADIATOR-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-ALL-RADIATOR-RUN-ALL-MODES | component | radiator | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-RADIATOR-RUN-ALL-MODES --output reports//CMP-ALL-RADIATOR-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-RADIATOR-SURFACE-CONTAMINATION | component | radiator | fault / surface_contamination | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-RADIATOR-SURFACE-CONTAMINATION --output reports//CMP-FLT-RADIATOR-SURFACE-CONTAMINATION.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-RADIATOR-DEPLOY-FAILURE | component | radiator | fault / deploy_failure | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-RADIATOR-DEPLOY-FAILURE --output reports//CMP-FLT-RADIATOR-DEPLOY-FAILURE.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-RADIATOR-EMISSIVITY-DECAY | component | radiator | degradation / emissivity_decay | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-RADIATOR-EMISSIVITY-DECAY --output reports//CMP-DEG-RADIATOR-EMISSIVITY-DECAY.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-RADIATOR-ABSORPTIVITY-INCREASE | component | radiator | degradation / absorptivity_increase | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-RADIATOR-ABSORPTIVITY-INCREASE --output reports//CMP-DEG-RADIATOR-ABSORPTIVITY-INCREASE.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-RADIATOR-FAULT-DEFAULT-SET | component | radiator | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-RADIATOR-FAULT-DEFAULT-SET --output reports//CMP-L2-RADIATOR-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-RADIATOR-DEGRADATION-DEFAULT-SET | component | radiator | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-RADIATOR-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-RADIATOR-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-RADIATOR-COMBINED-DEFAULT-SET | component | radiator | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-RADIATOR-COMBINED-DEFAULT-SET --output reports//CMP-L2-RADIATOR-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-RADIATOR-SHORT-DURATION-60S | component | radiator | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-RADIATOR-SHORT-DURATION-60S --output reports//CMP-BND-RADIATOR-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-RADIATOR-LONG-DURATION-86400S | component | radiator | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-RADIATOR-LONG-DURATION-86400S --output reports//CMP-BND-RADIATOR-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-RADIATOR-UNSUPPORTED-WORKING-SCENARIO | component | radiator | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-RADIATOR-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-RADIATOR-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-RADIATOR-REPEAT-STABILITY | component | radiator | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-RADIATOR-REPEAT-STABILITY --output reports//CMP-DET-RADIATOR-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-REACTION-WHEEL-NOMINAL | component | reaction_wheel | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-REACTION-WHEEL-NOMINAL --output reports//CMP-NOM-REACTION-WHEEL-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-ALL-REACTION-WHEEL-RUN-ALL-MODES | component | reaction_wheel | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-REACTION-WHEEL-RUN-ALL-MODES --output reports//CMP-ALL-REACTION-WHEEL-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-REACTION-WHEEL-BEARING-SEIZURE | component | reaction_wheel | fault / bearing_seizure | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-REACTION-WHEEL-BEARING-SEIZURE --output reports//CMP-FLT-REACTION-WHEEL-BEARING-SEIZURE.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-REACTION-WHEEL-MOTOR-OPEN | component | reaction_wheel | fault / motor_open | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-REACTION-WHEEL-MOTOR-OPEN --output reports//CMP-FLT-REACTION-WHEEL-MOTOR-OPEN.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-REACTION-WHEEL-BEARING-FRICTION-GROWTH | component | reaction_wheel | degradation / bearing_friction_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-REACTION-WHEEL-BEARING-FRICTION-GROWTH --output reports//CMP-DEG-REACTION-WHEEL-BEARING-FRICTION-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-REACTION-WHEEL-TORQUE-CONSTANT-DRIFT | component | reaction_wheel | degradation / torque_constant_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-REACTION-WHEEL-TORQUE-CONSTANT-DRIFT --output reports//CMP-DEG-REACTION-WHEEL-TORQUE-CONSTANT-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-REACTION-WHEEL-FAULT-DEFAULT-SET | component | reaction_wheel | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-REACTION-WHEEL-FAULT-DEFAULT-SET --output reports//CMP-L2-REACTION-WHEEL-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-REACTION-WHEEL-DEGRADATION-DEFAULT-SET | component | reaction_wheel | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-REACTION-WHEEL-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-REACTION-WHEEL-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-REACTION-WHEEL-COMBINED-DEFAULT-SET | component | reaction_wheel | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-REACTION-WHEEL-COMBINED-DEFAULT-SET --output reports//CMP-L2-REACTION-WHEEL-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-REACTION-WHEEL-SHORT-DURATION-60S | component | reaction_wheel | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-REACTION-WHEEL-SHORT-DURATION-60S --output reports//CMP-BND-REACTION-WHEEL-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-REACTION-WHEEL-LONG-DURATION-86400S | component | reaction_wheel | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-REACTION-WHEEL-LONG-DURATION-86400S --output reports//CMP-BND-REACTION-WHEEL-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-REACTION-WHEEL-UNSUPPORTED-WORKING-SCENARIO | component | reaction_wheel | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-REACTION-WHEEL-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-REACTION-WHEEL-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-REACTION-WHEEL-REPEAT-STABILITY | component | reaction_wheel | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-REACTION-WHEEL-REPEAT-STABILITY --output reports//CMP-DET-REACTION-WHEEL-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-SOLAR-PANEL-NOMINAL | component | solar_panel | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-SOLAR-PANEL-NOMINAL --output reports//CMP-NOM-SOLAR-PANEL-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-ALL-SOLAR-PANEL-RUN-ALL-MODES | component | solar_panel | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-SOLAR-PANEL-RUN-ALL-MODES --output reports//CMP-ALL-SOLAR-PANEL-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-SOLAR-PANEL-STRING-OPEN | component | solar_panel | fault / string_open | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-SOLAR-PANEL-STRING-OPEN --output reports//CMP-FLT-SOLAR-PANEL-STRING-OPEN.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-SOLAR-PANEL-SHADING | component | solar_panel | fault / shading | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-SOLAR-PANEL-SHADING --output reports//CMP-FLT-SOLAR-PANEL-SHADING.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-SOLAR-PANEL-RADIATION-EFFICIENCY-LOSS | component | solar_panel | degradation / radiation_efficiency_loss | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-SOLAR-PANEL-RADIATION-EFFICIENCY-LOSS --output reports//CMP-DEG-SOLAR-PANEL-RADIATION-EFFICIENCY-LOSS.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-SOLAR-PANEL-COVER-GLASS-DARKENING | component | solar_panel | degradation / cover_glass_darkening | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-SOLAR-PANEL-COVER-GLASS-DARKENING --output reports//CMP-DEG-SOLAR-PANEL-COVER-GLASS-DARKENING.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-SOLAR-PANEL-FAULT-DEFAULT-SET | component | solar_panel | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-SOLAR-PANEL-FAULT-DEFAULT-SET --output reports//CMP-L2-SOLAR-PANEL-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-SOLAR-PANEL-DEGRADATION-DEFAULT-SET | component | solar_panel | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-SOLAR-PANEL-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-SOLAR-PANEL-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-SOLAR-PANEL-COMBINED-DEFAULT-SET | component | solar_panel | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-SOLAR-PANEL-COMBINED-DEFAULT-SET --output reports//CMP-L2-SOLAR-PANEL-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-SOLAR-PANEL-SHORT-DURATION-60S | component | solar_panel | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-SOLAR-PANEL-SHORT-DURATION-60S --output reports//CMP-BND-SOLAR-PANEL-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-SOLAR-PANEL-LONG-DURATION-86400S | component | solar_panel | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-SOLAR-PANEL-LONG-DURATION-86400S --output reports//CMP-BND-SOLAR-PANEL-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-SOLAR-PANEL-UNSUPPORTED-WORKING-SCENARIO | component | solar_panel | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-SOLAR-PANEL-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-SOLAR-PANEL-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-SOLAR-PANEL-REPEAT-STABILITY | component | solar_panel | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-SOLAR-PANEL-REPEAT-STABILITY --output reports//CMP-DET-SOLAR-PANEL-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-STAR-TRACKER-NOMINAL | component | star_tracker | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-STAR-TRACKER-NOMINAL --output reports//CMP-NOM-STAR-TRACKER-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-STAR-TRACKER-NOMINAL | component | star_tracker | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-STAR-TRACKER-NOMINAL --output reports//CMP-WRK-STAR-TRACKER-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-STAR-TRACKER-STABLE-ATTITUDE | component | star_tracker | normal_working / stable_attitude | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-STAR-TRACKER-STABLE-ATTITUDE --output reports//CMP-WRK-STAR-TRACKER-STABLE-ATTITUDE.json | {"finite_numeric_required":true,"scenario":"stable_attitude"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-STAR-TRACKER-SLEW-PROFILE | component | star_tracker | normal_working / slew_profile | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-STAR-TRACKER-SLEW-PROFILE --output reports//CMP-WRK-STAR-TRACKER-SLEW-PROFILE.json | {"finite_numeric_required":true,"scenario":"slew_profile"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-STAR-TRACKER-LONG-DRIFT | component | star_tracker | normal_working / long_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-STAR-TRACKER-LONG-DRIFT --output reports//CMP-WRK-STAR-TRACKER-LONG-DRIFT.json | {"finite_numeric_required":true,"scenario":"long_drift"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-STAR-TRACKER-RUN-ALL-MODES | component | star_tracker | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-STAR-TRACKER-RUN-ALL-MODES --output reports//CMP-ALL-STAR-TRACKER-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-STAR-TRACKER-BLINDING | component | star_tracker | fault / blinding | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-STAR-TRACKER-BLINDING --output reports//CMP-FLT-STAR-TRACKER-BLINDING.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-STAR-TRACKER-DROPOUT | component | star_tracker | fault / dropout | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-STAR-TRACKER-DROPOUT --output reports//CMP-FLT-STAR-TRACKER-DROPOUT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-STAR-TRACKER-CENTROID-NOISE-GROWTH | component | star_tracker | degradation / centroid_noise_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-STAR-TRACKER-CENTROID-NOISE-GROWTH --output reports//CMP-DEG-STAR-TRACKER-CENTROID-NOISE-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-STAR-TRACKER-BORESIGHT-DRIFT | component | star_tracker | degradation / boresight_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-STAR-TRACKER-BORESIGHT-DRIFT --output reports//CMP-DEG-STAR-TRACKER-BORESIGHT-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-STAR-TRACKER-FAULT-DEFAULT-SET | component | star_tracker | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-STAR-TRACKER-FAULT-DEFAULT-SET --output reports//CMP-L2-STAR-TRACKER-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-STAR-TRACKER-DEGRADATION-DEFAULT-SET | component | star_tracker | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-STAR-TRACKER-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-STAR-TRACKER-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-STAR-TRACKER-COMBINED-DEFAULT-SET | component | star_tracker | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-STAR-TRACKER-COMBINED-DEFAULT-SET --output reports//CMP-L2-STAR-TRACKER-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-STAR-TRACKER-SHORT-DURATION-60S | component | star_tracker | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-STAR-TRACKER-SHORT-DURATION-60S --output reports//CMP-BND-STAR-TRACKER-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-STAR-TRACKER-LONG-DURATION-86400S | component | star_tracker | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-STAR-TRACKER-LONG-DURATION-86400S --output reports//CMP-BND-STAR-TRACKER-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-STAR-TRACKER-UNSUPPORTED-WORKING-SCENARIO | component | star_tracker | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-STAR-TRACKER-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-STAR-TRACKER-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-STAR-TRACKER-REPEAT-STABILITY | component | star_tracker | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-STAR-TRACKER-REPEAT-STABILITY --output reports//CMP-DET-STAR-TRACKER-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-SUN-SENSOR-NOMINAL | component | sun_sensor | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-SUN-SENSOR-NOMINAL --output reports//CMP-NOM-SUN-SENSOR-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-SUN-SENSOR-NOMINAL | component | sun_sensor | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-SUN-SENSOR-NOMINAL --output reports//CMP-WRK-SUN-SENSOR-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-SUN-SENSOR-FULL-SUN | component | sun_sensor | normal_working / full_sun | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-SUN-SENSOR-FULL-SUN --output reports//CMP-WRK-SUN-SENSOR-FULL-SUN.json | {"finite_numeric_required":true,"scenario":"full_sun"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-SUN-SENSOR-PARTIAL-SHADOW | component | sun_sensor | normal_working / partial_shadow | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-SUN-SENSOR-PARTIAL-SHADOW --output reports//CMP-WRK-SUN-SENSOR-PARTIAL-SHADOW.json | {"finite_numeric_required":true,"scenario":"partial_shadow"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-SUN-SENSOR-ECLIPSE | component | sun_sensor | normal_working / eclipse | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-SUN-SENSOR-ECLIPSE --output reports//CMP-WRK-SUN-SENSOR-ECLIPSE.json | {"finite_numeric_required":true,"scenario":"eclipse"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-SUN-SENSOR-RUN-ALL-MODES | component | sun_sensor | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-SUN-SENSOR-RUN-ALL-MODES --output reports//CMP-ALL-SUN-SENSOR-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-SUN-SENSOR-FALSE-ECLIPSE | component | sun_sensor | fault / false_eclipse | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-SUN-SENSOR-FALSE-ECLIPSE --output reports//CMP-FLT-SUN-SENSOR-FALSE-ECLIPSE.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-SUN-SENSOR-CELL-FAILURE | component | sun_sensor | fault / cell_failure | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-SUN-SENSOR-CELL-FAILURE --output reports//CMP-FLT-SUN-SENSOR-CELL-FAILURE.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-SUN-SENSOR-SENSITIVITY-DECAY | component | sun_sensor | degradation / sensitivity_decay | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-SUN-SENSOR-SENSITIVITY-DECAY --output reports//CMP-DEG-SUN-SENSOR-SENSITIVITY-DECAY.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-SUN-SENSOR-BIAS-DRIFT | component | sun_sensor | degradation / bias_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-SUN-SENSOR-BIAS-DRIFT --output reports//CMP-DEG-SUN-SENSOR-BIAS-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-SUN-SENSOR-FAULT-DEFAULT-SET | component | sun_sensor | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-SUN-SENSOR-FAULT-DEFAULT-SET --output reports//CMP-L2-SUN-SENSOR-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-SUN-SENSOR-DEGRADATION-DEFAULT-SET | component | sun_sensor | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-SUN-SENSOR-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-SUN-SENSOR-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-SUN-SENSOR-COMBINED-DEFAULT-SET | component | sun_sensor | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-SUN-SENSOR-COMBINED-DEFAULT-SET --output reports//CMP-L2-SUN-SENSOR-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-SUN-SENSOR-SHORT-DURATION-60S | component | sun_sensor | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-SUN-SENSOR-SHORT-DURATION-60S --output reports//CMP-BND-SUN-SENSOR-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-SUN-SENSOR-LONG-DURATION-86400S | component | sun_sensor | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-SUN-SENSOR-LONG-DURATION-86400S --output reports//CMP-BND-SUN-SENSOR-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-SUN-SENSOR-UNSUPPORTED-WORKING-SCENARIO | component | sun_sensor | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-SUN-SENSOR-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-SUN-SENSOR-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-SUN-SENSOR-REPEAT-STABILITY | component | sun_sensor | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-SUN-SENSOR-REPEAT-STABILITY --output reports//CMP-DET-SUN-SENSOR-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-THERMAL-NODE-NOMINAL | component | thermal_node | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-THERMAL-NODE-NOMINAL --output reports//CMP-NOM-THERMAL-NODE-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-THERMAL-NODE-NOMINAL | component | thermal_node | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-THERMAL-NODE-NOMINAL --output reports//CMP-WRK-THERMAL-NODE-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-THERMAL-NODE-COLD-SOAK-RECOVERY | component | thermal_node | normal_working / cold_soak_recovery | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-THERMAL-NODE-COLD-SOAK-RECOVERY --output reports//CMP-WRK-THERMAL-NODE-COLD-SOAK-RECOVERY.json | {"finite_numeric_required":true,"scenario":"cold_soak_recovery"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-THERMAL-NODE-PAYLOAD-HEATING | component | thermal_node | normal_working / payload_heating | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-THERMAL-NODE-PAYLOAD-HEATING --output reports//CMP-WRK-THERMAL-NODE-PAYLOAD-HEATING.json | {"finite_numeric_required":true,"scenario":"payload_heating"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-THERMAL-NODE-COOLDOWN | component | thermal_node | normal_working / cooldown | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-THERMAL-NODE-COOLDOWN --output reports//CMP-WRK-THERMAL-NODE-COOLDOWN.json | {"finite_numeric_required":true,"scenario":"cooldown"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-THERMAL-NODE-RUN-ALL-MODES | component | thermal_node | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-THERMAL-NODE-RUN-ALL-MODES --output reports//CMP-ALL-THERMAL-NODE-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-THERMAL-NODE-THERMAL-RUNAWAY | component | thermal_node | fault / thermal_runaway | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-THERMAL-NODE-THERMAL-RUNAWAY --output reports//CMP-FLT-THERMAL-NODE-THERMAL-RUNAWAY.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-THERMAL-NODE-SENSOR-BIAS | component | thermal_node | fault / sensor_bias | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-THERMAL-NODE-SENSOR-BIAS --output reports//CMP-FLT-THERMAL-NODE-SENSOR-BIAS.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-THERMAL-NODE-THERMAL-RESISTANCE-GROWTH | component | thermal_node | degradation / thermal_resistance_growth | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-THERMAL-NODE-THERMAL-RESISTANCE-GROWTH --output reports//CMP-DEG-THERMAL-NODE-THERMAL-RESISTANCE-GROWTH.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-THERMAL-NODE-HEAT-CAPACITY-DRIFT | component | thermal_node | degradation / heat_capacity_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-THERMAL-NODE-HEAT-CAPACITY-DRIFT --output reports//CMP-DEG-THERMAL-NODE-HEAT-CAPACITY-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-THERMAL-NODE-FAULT-DEFAULT-SET | component | thermal_node | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-THERMAL-NODE-FAULT-DEFAULT-SET --output reports//CMP-L2-THERMAL-NODE-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-THERMAL-NODE-DEGRADATION-DEFAULT-SET | component | thermal_node | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-THERMAL-NODE-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-THERMAL-NODE-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-THERMAL-NODE-COMBINED-DEFAULT-SET | component | thermal_node | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-THERMAL-NODE-COMBINED-DEFAULT-SET --output reports//CMP-L2-THERMAL-NODE-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-THERMAL-NODE-SHORT-DURATION-60S | component | thermal_node | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-THERMAL-NODE-SHORT-DURATION-60S --output reports//CMP-BND-THERMAL-NODE-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-THERMAL-NODE-LONG-DURATION-86400S | component | thermal_node | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-THERMAL-NODE-LONG-DURATION-86400S --output reports//CMP-BND-THERMAL-NODE-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-THERMAL-NODE-UNSUPPORTED-WORKING-SCENARIO | component | thermal_node | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-THERMAL-NODE-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-THERMAL-NODE-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-THERMAL-NODE-REPEAT-STABILITY | component | thermal_node | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-THERMAL-NODE-REPEAT-STABILITY --output reports//CMP-DET-THERMAL-NODE-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-THRUSTER-NOMINAL | component | thruster | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-THRUSTER-NOMINAL --output reports//CMP-NOM-THRUSTER-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-THRUSTER-NOMINAL | component | thruster | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-THRUSTER-NOMINAL --output reports//CMP-WRK-THRUSTER-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-THRUSTER-SHORT-IMPULSE | component | thruster | normal_working / short_impulse | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-THRUSTER-SHORT-IMPULSE --output reports//CMP-WRK-THRUSTER-SHORT-IMPULSE.json | {"finite_numeric_required":true,"scenario":"short_impulse"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-THRUSTER-STATION-KEEPING | component | thruster | normal_working / station_keeping | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-THRUSTER-STATION-KEEPING --output reports//CMP-WRK-THRUSTER-STATION-KEEPING.json | {"finite_numeric_required":true,"scenario":"station_keeping"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-THRUSTER-LONG-BURN | component | thruster | normal_working / long_burn | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-THRUSTER-LONG-BURN --output reports//CMP-WRK-THRUSTER-LONG-BURN.json | {"finite_numeric_required":true,"scenario":"long_burn"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-THRUSTER-RUN-ALL-MODES | component | thruster | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-THRUSTER-RUN-ALL-MODES --output reports//CMP-ALL-THRUSTER-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-THRUSTER-VALVE-STUCK-CLOSED | component | thruster | fault / valve_stuck_closed | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-THRUSTER-VALVE-STUCK-CLOSED --output reports//CMP-FLT-THRUSTER-VALVE-STUCK-CLOSED.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-THRUSTER-VALVE-STUCK-OPEN | component | thruster | fault / valve_stuck_open | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-THRUSTER-VALVE-STUCK-OPEN --output reports//CMP-FLT-THRUSTER-VALVE-STUCK-OPEN.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-THRUSTER-ISP-DECAY | component | thruster | degradation / isp_decay | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-THRUSTER-ISP-DECAY --output reports//CMP-DEG-THRUSTER-ISP-DECAY.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-THRUSTER-THRUST-COEFFICIENT-DRIFT | component | thruster | degradation / thrust_coefficient_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-THRUSTER-THRUST-COEFFICIENT-DRIFT --output reports//CMP-DEG-THRUSTER-THRUST-COEFFICIENT-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-THRUSTER-FAULT-DEFAULT-SET | component | thruster | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-THRUSTER-FAULT-DEFAULT-SET --output reports//CMP-L2-THRUSTER-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-THRUSTER-DEGRADATION-DEFAULT-SET | component | thruster | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-THRUSTER-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-THRUSTER-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-THRUSTER-COMBINED-DEFAULT-SET | component | thruster | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-THRUSTER-COMBINED-DEFAULT-SET --output reports//CMP-L2-THRUSTER-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-THRUSTER-SHORT-DURATION-60S | component | thruster | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-THRUSTER-SHORT-DURATION-60S --output reports//CMP-BND-THRUSTER-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-THRUSTER-LONG-DURATION-86400S | component | thruster | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-THRUSTER-LONG-DURATION-86400S --output reports//CMP-BND-THRUSTER-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-THRUSTER-UNSUPPORTED-WORKING-SCENARIO | component | thruster | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-THRUSTER-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-THRUSTER-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-THRUSTER-REPEAT-STABILITY | component | thruster | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-THRUSTER-REPEAT-STABILITY --output reports//CMP-DET-THRUSTER-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| CMP-NOM-TRANSMITTER-NOMINAL | component | transmitter | nominal / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-NOM-TRANSMITTER-NOMINAL --output reports//CMP-NOM-TRANSMITTER-NOMINAL.json | {"finite_numeric_required":true,"error_key_forbidden":true} | run_nominal_case returns structured data without error and all numeric outputs are finite. |
| CMP-WRK-TRANSMITTER-NOMINAL | component | transmitter | normal_working / nominal | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-TRANSMITTER-NOMINAL --output reports//CMP-WRK-TRANSMITTER-NOMINAL.json | {"finite_numeric_required":true,"scenario":"nominal"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-TRANSMITTER-TWT-DOWNLINK | component | transmitter | normal_working / twt_downlink | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-TRANSMITTER-TWT-DOWNLINK --output reports//CMP-WRK-TRANSMITTER-TWT-DOWNLINK.json | {"finite_numeric_required":true,"scenario":"twt_downlink"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-TRANSMITTER-SSPA-DOWNLINK | component | transmitter | normal_working / sspa_downlink | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-TRANSMITTER-SSPA-DOWNLINK --output reports//CMP-WRK-TRANSMITTER-SSPA-DOWNLINK.json | {"finite_numeric_required":true,"scenario":"sspa_downlink"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-WRK-TRANSMITTER-BURST-CONTACT | component | transmitter | normal_working / burst_contact | python scripts/run_engineering_verification_matrix.py --test-id CMP-WRK-TRANSMITTER-BURST-CONTACT --output reports//CMP-WRK-TRANSMITTER-BURST-CONTACT.json | {"finite_numeric_required":true,"scenario":"burst_contact"} | Named normal operating scenario executes and returns finite outputs with no error field. |
| CMP-ALL-TRANSMITTER-RUN-ALL-MODES | component | transmitter | batch / run_all_modes | python scripts/run_engineering_verification_matrix.py --test-id CMP-ALL-TRANSMITTER-RUN-ALL-MODES --output reports//CMP-ALL-TRANSMITTER-RUN-ALL-MODES.json | {"minimum_top_level_modes":3,"finite_numeric_required":true} | run_all_modes returns a dict with nominal/fault/degradation or L2-light coverage and finite numeric outputs. |
| CMP-FLT-TRANSMITTER-POWER-AMPLIFIER-FAULT | component | transmitter | fault / power_amplifier_fault | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-TRANSMITTER-POWER-AMPLIFIER-FAULT --output reports//CMP-FLT-TRANSMITTER-POWER-AMPLIFIER-FAULT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-FLT-TRANSMITTER-FREQUENCY-DRIFT-FAULT | component | transmitter | fault / frequency_drift_fault | python scripts/run_engineering_verification_matrix.py --test-id CMP-FLT-TRANSMITTER-FREQUENCY-DRIFT-FAULT --output reports//CMP-FLT-TRANSMITTER-FREQUENCY-DRIFT-FAULT.json | {"active_fault_required":true,"sample_window_s":[0,600]} | The single fault is active in its configured time window and changes effective parameters or output evidence. |
| CMP-DEG-TRANSMITTER-OUTPUT-POWER-DECAY | component | transmitter | degradation / output_power_decay | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-TRANSMITTER-OUTPUT-POWER-DECAY --output reports//CMP-DEG-TRANSMITTER-OUTPUT-POWER-DECAY.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-DEG-TRANSMITTER-FREQUENCY-STABILITY-DRIFT | component | transmitter | degradation / frequency_stability_drift | python scripts/run_engineering_verification_matrix.py --test-id CMP-DEG-TRANSMITTER-FREQUENCY-STABILITY-DRIFT --output reports//CMP-DEG-TRANSMITTER-FREQUENCY-STABILITY-DRIFT.json | {"active_degradation_required":true,"sample_window_s":[0,600]} | The single degradation is active after onset and changes effective parameters or output evidence. |
| CMP-L2-TRANSMITTER-FAULT-DEFAULT-SET | component | transmitter | fault_default_set / fault_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-TRANSMITTER-FAULT-DEFAULT-SET --output reports//CMP-L2-TRANSMITTER-FAULT-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default faults participate at least once. |
| CMP-L2-TRANSMITTER-DEGRADATION-DEFAULT-SET | component | transmitter | degradation_default_set / degradation_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-TRANSMITTER-DEGRADATION-DEFAULT-SET --output reports//CMP-L2-TRANSMITTER-DEGRADATION-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | All default degradations participate at least once. |
| CMP-L2-TRANSMITTER-COMBINED-DEFAULT-SET | component | transmitter | combined_default_set / combined_default_set | python scripts/run_engineering_verification_matrix.py --test-id CMP-L2-TRANSMITTER-COMBINED-DEFAULT-SET --output reports//CMP-L2-TRANSMITTER-COMBINED-DEFAULT-SET.json | {"duration_s":600,"finite_numeric_required":true} | At least one fault and one degradation participate in one run. |
| CMP-BND-TRANSMITTER-SHORT-DURATION-60S | component | transmitter | duration_boundary / short_duration_60s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-TRANSMITTER-SHORT-DURATION-60S --output reports//CMP-BND-TRANSMITTER-SHORT-DURATION-60S.json | {"duration_s":60.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-BND-TRANSMITTER-LONG-DURATION-86400S | component | transmitter | duration_boundary / long_duration_86400s | python scripts/run_engineering_verification_matrix.py --test-id CMP-BND-TRANSMITTER-LONG-DURATION-86400S --output reports//CMP-BND-TRANSMITTER-LONG-DURATION-86400S.json | {"duration_s":86400.0,"finite_numeric_required":true} | Boundary-duration combined sparse event run completes with finite outputs and bounded sample count. |
| CMP-NEG-TRANSMITTER-UNSUPPORTED-WORKING-SCENARIO | component | transmitter | negative / unsupported_working_scenario | python scripts/run_engineering_verification_matrix.py --test-id CMP-NEG-TRANSMITTER-UNSUPPORTED-WORKING-SCENARIO --output reports//CMP-NEG-TRANSMITTER-UNSUPPORTED-WORKING-SCENARIO.json | {"expected_exception":"ValueError"} | Unsupported working scenario is rejected by the component runner. |
| CMP-DET-TRANSMITTER-REPEAT-STABILITY | component | transmitter | determinism / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id CMP-DET-TRANSMITTER-REPEAT-STABILITY --output reports//CMP-DET-TRANSMITTER-REPEAT-STABILITY.json | {"repeat_count":2,"semantic_hash_equal":true} | Two same-input runs produce the same semantic hash. |
| SUB-ADCS-ALL-MODES | subsystem | adcs | all_modes / all_modes | python scripts/run_engineering_verification_matrix.py --test-id SUB-ADCS-ALL-MODES --output reports//SUB-ADCS-ALL-MODES.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-ADCS-REPEAT-STABILITY | subsystem | adcs | repeat_stability / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id SUB-ADCS-REPEAT-STABILITY --output reports//SUB-ADCS-REPEAT-STABILITY.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-COMM-DATA-ALL-MODES | subsystem | comm_data | all_modes / all_modes | python scripts/run_engineering_verification_matrix.py --test-id SUB-COMM-DATA-ALL-MODES --output reports//SUB-COMM-DATA-ALL-MODES.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-COMM-DATA-REPEAT-STABILITY | subsystem | comm_data | repeat_stability / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id SUB-COMM-DATA-REPEAT-STABILITY --output reports//SUB-COMM-DATA-REPEAT-STABILITY.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-EPS-ALL-MODES | subsystem | eps | all_modes / all_modes | python scripts/run_engineering_verification_matrix.py --test-id SUB-EPS-ALL-MODES --output reports//SUB-EPS-ALL-MODES.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-EPS-REPEAT-STABILITY | subsystem | eps | repeat_stability / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id SUB-EPS-REPEAT-STABILITY --output reports//SUB-EPS-REPEAT-STABILITY.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-PAYLOAD-ALL-MODES | subsystem | payload | all_modes / all_modes | python scripts/run_engineering_verification_matrix.py --test-id SUB-PAYLOAD-ALL-MODES --output reports//SUB-PAYLOAD-ALL-MODES.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-PAYLOAD-REPEAT-STABILITY | subsystem | payload | repeat_stability / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id SUB-PAYLOAD-REPEAT-STABILITY --output reports//SUB-PAYLOAD-REPEAT-STABILITY.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-PROPULSION-ALL-MODES | subsystem | propulsion | all_modes / all_modes | python scripts/run_engineering_verification_matrix.py --test-id SUB-PROPULSION-ALL-MODES --output reports//SUB-PROPULSION-ALL-MODES.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-PROPULSION-REPEAT-STABILITY | subsystem | propulsion | repeat_stability / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id SUB-PROPULSION-REPEAT-STABILITY --output reports//SUB-PROPULSION-REPEAT-STABILITY.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-THERMAL-ALL-MODES | subsystem | thermal | all_modes / all_modes | python scripts/run_engineering_verification_matrix.py --test-id SUB-THERMAL-ALL-MODES --output reports//SUB-THERMAL-ALL-MODES.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| SUB-THERMAL-REPEAT-STABILITY | subsystem | thermal | repeat_stability / repeat_stability | python scripts/run_engineering_verification_matrix.py --test-id SUB-THERMAL-REPEAT-STABILITY --output reports//SUB-THERMAL-REPEAT-STABILITY.json | {"finite_numeric_required":true} | Subsystem public runner completes, returns structured outputs and contains finite numeric values. |
| WHO-REP-001 | whole_spacecraft_or_agent_release | component_nominal | representative_run / component_nominal | sat-agent run examples/component_nominal.yaml --output-root runs//WHO-REP-001 | {"expected_four_state":true,"input":"examples/component_nominal.yaml"} | Run result matches the documented four-state expectation for the representative case. |
| WHO-REP-002 | whole_spacecraft_or_agent_release | component_fault | representative_run / component_fault | sat-agent run examples/component_fault.yaml --output-root runs//WHO-REP-002 | {"expected_four_state":true,"input":"examples/component_fault.yaml"} | Run result matches the documented four-state expectation for the representative case. |
| WHO-REP-003 | whole_spacecraft_or_agent_release | component_degradation | representative_run / component_degradation | sat-agent run examples/component_degradation.yaml --output-root runs//WHO-REP-003 | {"expected_four_state":true,"input":"examples/component_degradation.yaml"} | Run result matches the documented four-state expectation for the representative case. |
| WHO-REP-004 | whole_spacecraft_or_agent_release | subsystem_nominal | representative_run / subsystem_nominal | sat-agent run examples/subsystem_nominal.yaml --output-root runs//WHO-REP-004 | {"expected_four_state":true,"input":"examples/subsystem_nominal.yaml"} | Run result matches the documented four-state expectation for the representative case. |
| WHO-REP-005 | whole_spacecraft_or_agent_release | whole_spacecraft_combined | representative_run / whole_spacecraft_combined | sat-agent run examples/whole_spacecraft_combined_nominal.yaml --output-root runs//WHO-REP-005 | {"expected_four_state":true,"input":"examples/whole_spacecraft_combined_nominal.yaml"} | Run result matches the documented four-state expectation for the representative case. |
| WHO-REP-006 | whole_spacecraft_or_agent_release | mission_fail_preserved | representative_run / mission_fail_preserved | sat-agent run examples/mission_fail_preserved.yaml --output-root runs//WHO-REP-006 | {"expected_four_state":true,"input":"examples/mission_fail_preserved.yaml"} | Run result matches the documented four-state expectation for the representative case. |
| ENV-001 | environment | environment | version / version | sat-agent version | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| ENV-002 | environment | environment | strict_doctor / strict_doctor | sat-agent doctor --require-api --strict-assets | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| STA-001 | static_quality | static_quality | compileall / compileall | python -m compileall -q src scripts tests | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| STA-002 | static_quality | static_quality | ruff_blockers / ruff_blockers | ruff check src tests scripts --select F821,F822,F811,F841 | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| STA-003 | static_quality | static_quality | duplicate_definitions / duplicate_definitions | python scripts/check_no_duplicate_module_definitions.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| STA-004 | static_quality | static_quality | public_exports / public_exports | python scripts/check_public_module_exports.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| STA-005 | static_quality | static_quality | public_type_hints / public_type_hints | python scripts/check_public_type_hints.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| STA-006 | static_quality | static_quality | exception_policy / exception_policy | python scripts/check_exception_policy_既有工程版本.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| CAL-001 | calibration | calibration | readiness_check / readiness_check | sat-agent calibration check --project-root . | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| CAL-002 | calibration | calibration | readiness_report / readiness_report | sat-agent calibration report --project-root . | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| CAL-003 | calibration | calibration | schema / schema | sat-agent calibration schema --kind all | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| CAL-004 | calibration | calibration | invalid_ground_calibrated_rejection / invalid_ground_calibrated_rejection | sat-agent calibration validate examples/calibration/invalid_ground_calibrated_missing_data.yaml --project-root . | {"expected_exit_codes":[1]} | Command exits with expected return code [1] and writes no unhandled exception trace. |
| MOD-001 | model_contract | model_contract | layer_schema / layer_schema | python scripts/check_layer_schema_contract.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| MOD-002 | model_contract | model_contract | native_capability_audit / native_capability_audit | python scripts/check_native_capability_audit_v2.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| MOD-003 | model_contract | model_contract | parameter_provenance / parameter_provenance | python scripts/check_parameter_provenance_calibration_1.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| MOD-004 | model_contract | model_contract | subsystem_builders / subsystem_builders | python scripts/check_subsystem_basilisk_builders.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| MOD-005 | model_contract | model_contract | subsystem_runners / subsystem_runners | python scripts/check_subsystem_basilisk_runners.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| MOD-006 | model_contract | model_contract | runtime_injection / runtime_injection | python scripts/check_runtime_injection_closure_2.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| MOD-007 | model_contract | model_contract | whole_spacecraft_native / whole_spacecraft_native | python scripts/check_wholesc_native_closure_1.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| AGT-001 | agent | agent | parse_template / parse_template | sat-agent parse '创建电池部件正常仿真，运行 300 秒' --backend template | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| AGT-002 | agent | agent | validate_component / validate_component | sat-agent validate examples/component_nominal.yaml | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| AGT-003 | agent | agent | resolve_component / resolve_component | sat-agent resolve examples/component_nominal.yaml | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| AGT-004 | agent | agent | plan_whole / plan_whole | sat-agent plan examples/whole_spacecraft_combined_nominal.yaml | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| AGT-005 | agent | agent | export_script / export_script | sat-agent export-script examples/component_nominal.yaml --output generated/既有工程版本_component_nominal.py | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| AGT-006 | agent | agent | golden_repeat5 / golden_repeat5 | sat-agent eval --repeat-count 5 --execute-marked --output-dir reports/既有工程版本_golden | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| REL-001 | release | release | pytest / pytest | python -m pytest -q | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |
| REL-002 | release | release | release_check / release_check | sat-agent release-check --golden-report reports/agent_golden/golden_eval_report.json --output-dir reports/既有工程版本_release_check --source-root . --strict-assets | {"expected_exit_codes":[0]} | Command exits with expected return code [0] and writes no unhandled exception trace. |

# 7. 失败项处置要求

| 失败类型 | 定位文件 | 处置要求 |
| --- | --- | --- |
| 组件运行失败 | reports//<TEST_ID>.json | 检查 result_preview、reason、traceback 和组件 runner。 |
| QoI 或数值非有限 | reports//<TEST_ID>.json | 检查 NaN/Inf 路径，禁止继续声明 PASS。 |
| 运行时注入未参与 | trace.active_faults / active_degradations | 检查事件时间窗、目标参数、default_faults/default_degradations。 |
| Shell命令失败 | stdout_tail/stderr_tail | 按返回码和 traceback 定位环境、资产或命令问题。 |
| Golden Set 失败 | reports/既有工程版本_golden | 检查 TaskSpec hash、Plan hash、Claim hash 和失败案例。 |

# 8. 归档要求

正式验收需归档：矩阵 JSON、每项测试 JSON、summary.json、Golden Set 报告、release-check 报告、doctor 报告、制品 SHA-256、Run Bundle 和失败项处置记录。